# Data Dictionary and Variable Codebook

Emergency Vehicle Routing Simulation Dataset

Project: Simultaneous Routing of Emergency Vehicles in Poorly Developed and Congested Urban Networks
Location: Zomba, Malawi
Author: Josophat Makawa (BSC/MAT/14/21)
Supervisor: Dr. E. Mwakilama
Institution: University of Malawi - Mathematical Sciences Department
Generated: February 2026

# 1. Introduction

This document provides a comprehensive description of all variables, data structures, and 
parameters used in the emergency vehicle routing simulation. The dataset is synthetic but designed to 
closely resemble real-world traffic dynamics in poorly planned, congested urban areas of developing 
countries, specifically Zomba, Malawi.

## 1.1 Methodology Framework

The simulation implements a multi-objective optimization approach that balances:
• Travel time (minimization)
• Route reliability (maximization)
• Operational cost (minimization)
• Network resilience (criticality awareness)

The model uses time-dependent travel times τij(t) with piecewise-linear congestion functions, 
edge betweenness centrality for criticality assessment, and sequential priority-based vehicle routing.

# 2. Dataset Overview

| Component | Description | Records |
| --- | --- | --- |
| Nodes | Network intersections, hospitals, and markets | 40 |
| Edges | Directed road segments with properties | 200 |
| Time Profiles | Congestion patterns throughout the day | 5 |
| Vehicles | Emergency ambulances | 5 |
| Incidents | Emergency calls requiring response | 15 |
| Operational Costs | Cost parameters for each edge | 200 |
| Reliability Parameters | Variability factors for reliability calculation | 200 |
| Time-Dependent Travel | Travel times for each edge by time period | 1000 |

# 3. Nodes (Network Vertices)

Nodes represent spatial points in the urban network including intersections, hospitals 
(ambulance bases), and market areas. Total nodes: 40.

| Variable | Type | Description | Values/Units |
| --- | --- | --- | --- |
| node_id | int | Unique identifier for each node | 1-40 |
| node_type | str | Category of node | intersection, hospital, market |
| x | float | Horizontal spatial coordinate | 0-10 km |
| y | float | Vertical spatial coordinate | 0-10 km |
| zone | str | Urban zone classification | CBD, residential, peri-urban |
| name | str | Human-readable identifier | e.g., Hospital_A, Market_1 |

Node Distribution: 3 hospitals (spatially separated), 4 markets (clustered near CBD), 33 intersections

# 4. Edges (Road Segments)

Edges represent directed road segments connecting nodes. Each edge has physical properties 
(length, speed) and temporal characteristics (congestion patterns). Total edges: 200.

| Variable | Type | Description | Values/Units |
| --- | --- | --- | --- |
| edge_id | int | Unique identifier for each edge | 1-200 |
| from_node | int | Origin node ID | References node_id |
| to_node | int | Destination node ID | References node_id |
| length_km | float | Physical length of road segment | Kilometers (km) |
| road_type | str | Classification of road | trunk, primary, residential |
| lanes | int | Number of traffic lanes | 1, 2, or 4 |
| free_speed_kmh | float | Speed limit / free-flow speed | Kilometers per hour |
| is_market_road | bool | Whether edge passes through market area | True/False |
| base_travel_time_min | float | Free-flow travel time (τ⁰ij) | Minutes |
| criticality_rho | float | Edge betweenness centrality (ρij) | 0-1 (normalized) |

Road Type Characteristics:
• Trunk: High-capacity roads, ~60 km/h, 4 lanes
• Primary: Main roads, ~40 km/h, 2 lanes
• Residential: Local streets, ~25 km/h, 1 lane

Criticality Score (ρij): Calculated using edge betweenness centrality - the fraction of all-pairs shortest 
paths that traverse this edge. Higher values indicate structural importance; closure of high-criticality 
edges significantly degrades network connectivity.

# 5. Time Profiles (Congestion Patterns)

Time profiles define how congestion varies throughout a 24-hour period. Different multipliers 
apply to normal roads versus market roads, reflecting the impact of roadside trading and market activity.

| Variable | Type | Description | Values/Units |
| --- | --- | --- | --- |
| time_period | str | Name of time period | Categorical label |
| start_hour | int | Beginning of period | 0-23 hours |
| end_hour | int | End of period | 1-24 hours |
| multiplier_normal | float | Congestion factor for normal roads | 1.0-2.5 |
| multiplier_market | float | Congestion factor for market roads | 1.0-2.8 |
| description | str | Qualitative description | Text |

Time Periods Defined:
1. Night (0:00-6:00): Free flow, multiplier = 1.0
2. Morning Peak (6:00-9:00): Heavy congestion, market roads especially affected
3. Midday (9:00-15:00): Moderate traffic, markets active
4. Evening Peak (15:00-19:00): Maximum congestion period
5. Late Evening (19:00-24:00): Reduced traffic as markets close

Actual Travel Time Calculation:
τij(t) = base_travel_time × multiplier
where multiplier is selected based on road type (normal/market) and current time period.

# 6. Vehicles (Emergency Ambulances)

Emergency vehicles (ambulances) are stationed at hospital nodes and dispatched to 
incidents based on priority. Total vehicles: 5.

| Variable | Type | Description | Values/Units |
| --- | --- | --- | --- |
| vehicle_id | str | Unique identifier | AMB_01 to AMB_05 |
| base_node | int | Home station (hospital) | References node_id |
| available_from | float | Time when vehicle becomes available | Hours (0-24) |
| status | str | Current operational status | available, dispatched, busy |
| priority_rank | int | Dispatch priority | 1-5 (1 = highest) |
| crew_size | int | Number of crew members | Persons |
| fuel_capacity_L | float | Fuel tank capacity | Liters |
| avg_fuel_consumption | float | Fuel efficiency | Liters per kilometer |

Vehicle Distribution: Vehicles are distributed across the 3 hospital nodes to ensure geographic coverage.

# 7. Incidents (Emergency Calls)

Emergency incidents represent calls requiring ambulance response. Incidents are distributed 
temporally (higher during peak hours) and spatially (higher in CBD/market areas). Total incidents: 15.

| Variable | Type | Description | Values/Units |
| --- | --- | --- | --- |
| incident_id | str | Unique identifier | INC_001 to INC_015 |
| node_id | int | Location of emergency | References node_id |
| call_time_hours | float | Time of call (decimal hours) | 0-24 hours |
| call_time_formatted | str | Time in HH:MM format | e.g., 08:45 |
| severity | str | Medical urgency level | critical, urgent, moderate, minor |
| priority | int | Dispatch priority (1=highest) | 1-4 |
| service_time_min | float | On-scene time required | Minutes |
| description | str | Brief description | Text |

Severity Classifications:
• Critical: Life-threatening, priority 1, service time 15-30 min (20% of incidents)
• Urgent: Serious but stable, priority 2, service time 10-20 min (35%)
• Moderate: Non-life-threatening, priority 3, service time 5-15 min (30%)
• Minor: Minor injury/illness, priority 4, service time 5-10 min (15%)

# 8. Operational Costs

Operational costs capture the economic dimension of routing decisions. Costs are computed 
for each edge based on fuel consumption, vehicle maintenance, and crew wages. Values in Malawian Kwacha (MWK).

| Variable | Type | Description | Values/Units |
| --- | --- | --- | --- |
| edge_id | int | Reference to edge | References edge_id |
| fuel_cost_MWK | float | Cost of fuel for traversal | Malawian Kwacha |
| maintenance_cost_MWK | float | Vehicle wear and tear cost | Malawian Kwacha |
| wage_cost_MWK | float | Crew compensation for travel time | Malawian Kwacha |
| total_cost_MWK | float | Sum of all cost components | Malawian Kwacha |
| cost_per_km_MWK | float | Normalized cost per kilometer | MWK per km |

Cost Parameters Used:
• Fuel price: 2,850 MWK/liter (approximate Malawi price)
• Fuel consumption: 0.12 L/km (ambulance average)
• Maintenance: 150 MWK/km (wear and tear)
• Crew wage: 5,000 MWK/hour (average emergency services wage)

Total Cost Formula:
total_cost = (length × 0.12 × 2850) + (length × 150) + (travel_time_hours × 5000)

# 9. Reliability Parameters

Reliability parameters quantify the variability and predictability of travel times 
on each edge. Higher variability indicates less reliable routes that may experience unpredictable delays.

| Variable | Type | Description | Values/Units |
| --- | --- | --- | --- |
| edge_id | int | Reference to edge | References edge_id |
| gamma | float | Variability factor (γ) | 0.15-0.60 (dimensionless) |
| reliability_class | str | Qualitative reliability rating | high, medium, low |

Gamma Values by Road Type:
• Trunk roads: γ = 0.15 (more predictable)
• Primary roads: γ = 0.25
• Residential roads: γ = 0.40 (higher variability)
• Market roads: γ multiplied by 1.5 (additional unpredictability)

Travel Time Variability Formula (vij):
vij(t) = τij(t) × [τij(t) / τ⁰ij]² × γ

This formula penalizes routes that become congested (τij >> τ⁰ij) and have high variability (large γ).

# 10. Time-Dependent Travel Times

This table expands the edge and time profile information into explicit travel times 
for each edge during each time period. Total records: 1,000 (200 edges × 5 time periods).

| Variable | Type | Description | Values/Units |
| --- | --- | --- | --- |
| edge_id | int | Reference to edge | References edge_id |
| from_node | int | Origin node | References node_id |
| to_node | int | Destination node | References node_id |
| time_period | str | Time period name | night, morning_peak, etc. |
| start_hour | int | Period start | 0-23 hours |
| end_hour | int | Period end | 1-24 hours |
| base_travel_time_min | float | Free-flow travel time | Minutes |
| congestion_multiplier | float | Applied congestion factor | 1.0-2.8 |
| actual_travel_time_min | float | Congested travel time | Minutes |

Usage: This table allows the routing algorithm to look up the expected travel time for any 
edge at any time of day. During simulation, the algorithm selects the appropriate time period based 
on the current simulation time and retrieves the actual_travel_time_min value.

# 11. Mathematical Notation Reference

Key mathematical symbols used in the thesis and simulation model.

| Symbol | Description | Dataset Variable |
| --- | --- | --- |
| G = (V, E) | Directed graph (network) | Nodes and Edges tables |
| τij(t) | Time-dependent travel time | actual_travel_time_min |
| τ⁰ij | Free-flow travel time | base_travel_time_min |
| ρij | Edge betweenness centrality (criticality) | criticality_rho |
| γ | Variability factor | gamma |
| vij(t) | Travel time variability | Calculated from γ and τij |
| cij | Operational cost | total_cost_MWK |
| ℓij | Edge length | length_km |
| s⁰ij | Free-flow speed | free_speed_kmh |

# 12. Multi-Objective Formulation

The routing model optimizes three objectives simultaneously for each path P from origin to destination:

Objective 1: Travel Time + Cost
f'₁(P) = Σ [τij(ti) + λ·cij]
Minimizes the sum of travel time and operational cost (converted to time-equivalent units via λ)

Objective 2: Reliability
f₂(P) = Σ vij(ti)
Minimizes the total travel time variability (unreliability) along the path

Objective 3: Resilience
f₃(P) = β·Σ ρij
Penalizes paths that rely heavily on critical edges (β ≥ 0 is a tunable parameter)

Solution Approach: The Multi-Objective Label-Setting (MOLS) algorithm generates a Pareto front of 
non-dominated solutions, allowing decision-makers to select routes based on the specific emergency context 
(e.g., prioritize speed for cardiac emergencies vs. resilience during disasters).

# 13. Data Generation Process

This synthetic dataset was generated using network science and stochastic modeling techniques:

1. Network Topology:
   • Nodes placed randomly on 10×10 km grid
   • Hospitals strategically positioned for coverage
   • Markets clustered near CBD center (5,5)
   • Edges created based on proximity (average degree ~4-5)

2. Edge Properties:
   • Road types assigned probabilistically (trunk for long distances)
   • Speeds vary by road type with ±10% noise
   • Market roads identified by proximity to market nodes

3. Criticality Calculation:
   • Edge betweenness centrality computed using NetworkX
   • Normalized to [0,1] range

4. Incident Generation:
   • Temporal distribution weighted toward peak hours
   • Spatial distribution weighted toward CBD/markets
   • Severity levels sampled from realistic distribution

5. Validation:
   • All formulas verified for consistency
   • Network connectivity confirmed
   • Parameter ranges validated against literature

# 14. Usage Guidelines

For Simulation:
1. Load all required tables (nodes, edges, time_dependent_travel, vehicles, incidents)
2. Initialize network graph G from nodes and edges
3. For each time-dependent query, select appropriate time period
4. Apply MOLS algorithm to find Pareto-optimal routes
5. Update vehicle availability and edge travel times after each dispatch

For Analysis:
• Use criticality_rho to identify vulnerable network points
• Compare performance across different time periods
• Evaluate trade-offs between speed, reliability, and cost
• Test resilience by simulating edge closures

For Visualization:
• Plot nodes using (x, y) coordinates
• Color-code by node_type or zone
• Draw edges with thickness proportional to criticality
• Animate vehicle movements using call_time and travel times

# 15. References and Further Reading

Thesis Document:
Makawa, J. (2025). Simultaneous Routing of Emergency Vehicles in Poorly Developed and Congested Urban Networks. BSc Thesis, University of Malawi.

Key Methodological Papers:
• Li et al. (2025): Multi-objective routing with demand urgency
• Luan & Jiang (2024): MIPSSTW model for complex traffic conditions
• Zhao et al. (2019): Multi-objective emergency vehicle routing
• Sun et al. (2024): Critical link identification in stochastic networks

Data Sources:
• OpenStreetMap (OSM): Network topology and road classifications
• Kalonde et al. (2025): Travel time mapping in Malawi
• World Bank (2025): Infrastructure conditions in developing countries

________________________________________________________________________________

End of Data Dictionary
University of Malawi - Mathematical Sciences Department - 2026